# Calculation Breakdown for BIPV Facade Area Results

## 1. What the Pipeline Calculates

For each street-level building image, the pipeline estimates:

- Number of floors

- Real-world facade height in metres

- Real-world facade width in metres

- Total facade area in square metres

- Usable BIPV facade area in square metres

- Percentage of facade usable for BIPV

- Percentage of facade excluded from BIPV

- Number of PV panels that can fit

- Installed PV capacity in kWp

- Estimated annual energy output in kWh

- JSON and Excel outputs for PVsyst-ready downstream use

The image-processing stages are:

- Facade image input

- Obstacle detection

- Obstacle removal / inpainting

- Facade alignment / rectification

- Facade segmentation result

- Area and PV capacity estimation

## 2. How the Number of Floors Is Obtained

The model detects windows and opening rows on the facade. These detected openings are grouped vertically into floor bands.

For example, if the detected window rows form four clear horizontal bands, the model estimates:

num_floors = 4

The result also records where the floor count came from:

floor_count_source

Typical values include:

dino_window_boxes
segmented_window_mask
segmented_window_mask_with_facade_extent

This means the model can count floors either from direct Grounding DINO window detections or from the final segmented window/opening mask.

## 3. How Facade Height Is Calculated

The real-world facade height is estimated using:

Facade height (m) = Number of floors × Estimated floor height (m)

The floor height is an architectural prior. In this project, the default urban value is usually:

3.3 m per floor

For buildings with smaller windows or high-rise characteristics, the model may use:

3.4 m per floor

For low-rise buildings with larger window proportions, it may use:

3.1 m per floor

Example for IMG_2000:

Number of floors = 4
Estimated floor height = 3.3 m

Facade height = 4 × 3.3
              = 13.2 m

Example for IMG_0000:

Number of floors = 15
Estimated floor height = 3.4 m

Facade height = 15 × 3.4
              = 51.0 m

## 4. How Pixel-to-Metre Scaling Is Obtained

After facade segmentation, the model knows the facade height in pixels:

facade_height_px

It also estimates the real-world height in metres:

height_m

The pixel-to-metre scale is:

pixels_per_meter = facade_height_px / height_m

This tells the model how many image pixels represent one metre in real-world facade height.

Example for IMG_2000:

pixels_per_meter = 35.1515 px/m

This means approximately 35.15 pixels in the rectified facade image represent 1 metre in real life.

## 5. How Facade Width Is Calculated

Once the model knows the pixel-to-metre scale, it converts facade width from pixels into metres:

Facade width (m) = facade_width_px / pixels_per_meter

The facade_width_px is obtained from the width of the segmented facade mask after rectification.

For example, IMG_2000 produced:

Facade width = 28.16 m

This means the segmented, rectified facade was estimated to represent a real-world width of approximately 28.16 metres.

## 6. How Total Facade Area Is Calculated

The total facade area is calculated using the standard rectangle area equation:

Total facade area (m²) = Facade height (m) × Facade width (m)

Example for IMG_2000:

Height = 13.20 m
Width  = 28.16 m

Total facade area = 13.20 × 28.16
                  = 371.76 m²

Example for IMG_2001:

Height = 19.80 m
Width  = 34.34 m

Total facade area = 19.80 × 34.34
                  = 679.84 m²

## 7. How the Mask Pixel Count Is Used

The segmentation output is a binary mask. Each pixel is either:

True  = belongs to the region
False = does not belong to the region

In Python, True behaves like 1 and False behaves like 0.

Therefore, the model counts pixels by summing the mask:

facade_area_px = facade_mask.sum()
usable_area_px = usable_mask.sum()

The facade mask contains all pixels classified as facade wall surface. The usable mask contains only the pixels that remain after excluding windows, doors, balconies, roof/opening areas, and other unavailable regions.

## 8. How Pixels Are Converted to Square Metres

The model first calculates how much real-world area one facade pixel represents:

m² per pixel = total facade area (m²) / facade area pixel count

In code this is:

px_to_m2 = dimensions["total_facade_area_m2"] / facade_area_px

Then the usable BIPV area is:

Usable area (m²) = usable mask pixel count × m² per pixel

In code this is:

usable_area_m2 = usable_mask.sum() * px_to_m2

This means the model does not guess the usable area directly. It:

- Segments the facade.

- Counts facade pixels.

- Converts the total facade into real-world square metres.

- Calculates the square-metre value of each pixel.

- Counts usable pixels.

- Converts usable pixels into square metres.

## 9. What Is Excluded From Usable BIPV Area

The usable BIPV area is the facade wall area remaining after exclusions.

The excluded regions include:

- Windows

- Doors

- Balconies

- Roof/opening masks where detected

- Architectural voids or unavailable facade regions

- Optional obstacle exclusion if enabled

The general logic is:

usable_mask = facade_mask
              - window_mask
              - door_mask
              - balcony_mask
              - roof/opening mask

The code also dilates some exclusions slightly to represent mounting clearance around openings.

## 10. How Usable and Excluded Percentages Are Calculated

Usable percentage:

Usable area (%) = usable area (m²) / total facade area (m²) × 100

Excluded area:

Excluded area (m²) = total facade area (m²) - usable area (m²)

Excluded percentage:

Excluded area (%) = 100 - usable area (%)

Example for IMG_2000:

Total facade area = 371.76 m²
Usable area       = 252.89 m²

Usable percentage = 252.89 / 371.76 × 100
                  = 68.03%

Excluded area     = 371.76 - 252.89
                  = 118.87 m²

Excluded percent  = 100 - 68.03
                  = 31.97%

## 11. How Panel Count Is Calculated

The project assumes:

Panel area = 1.7 m²
Panel power = 350 W

The number of panels is calculated as:

Number of panels = floor(usable area / panel area)

The floor operation means only complete panels are counted.

Example for IMG_2000:

Usable area = 252.89 m²
Panel area  = 1.7 m²

Panels = floor(252.89 / 1.7)
       = floor(148.76)
       = 148 panels

## 12. How Installed Capacity Is Calculated

Each panel is assumed to be:

350 W = 0.35 kW

Installed capacity is:

Total capacity (kW) = number of panels × watts per panel / 1000

Example for IMG_2000:

Panels = 148
Watts per panel = 350 W

Capacity = 148 × 350 / 1000
         = 51.8 kW

## 13. How Annual Energy Output Is Calculated

The project uses a specific yield assumption:

Specific yield = 950 kWh/kWp/year

Annual energy is calculated as:

Annual energy (kWh) = installed capacity (kWp) × specific yield

Example for IMG_2000:

Installed capacity = 51.8 kWp
Specific yield = 950 kWh/kWp/year

Annual energy = 51.8 × 950
              = 49,210 kWh/year

This is a simplified PV yield estimate. It is useful for screening and comparison, but a full PVsyst simulation would later refine it using orientation, climate data, shading, system losses, and module parameters.

## 14. Batch Result Table

| Building | Floors | Height (m) | Width (m) | Total facade area (m²) | Segmented usable area (m²) | Usable area (%) | Excluded area (m²) | Excluded area (%) | Panels | Capacity (kWp) | Annual energy (kWh) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IMG_2000 | 4 | 13.20 | 28.16 | 371.76 | 252.89 | 68.03 | 118.87 | 31.97 | 148 | 51.80 | 49,210.0 |
| IMG_2001 | 6 | 19.80 | 34.34 | 679.84 | 519.16 | 76.36 | 160.68 | 23.64 | 305 | 106.75 | 101,412.5 |
| IMG_2222 | 4 | 13.20 | 21.48 | 283.51 | 216.92 | 76.51 | 66.59 | 23.49 | 127 | 44.45 | 42,227.5 |
| IMG_6387 | 4 | 12.40 | 18.19 | 225.54 | 179.43 | 79.55 | 46.11 | 20.45 | 105 | 36.75 | 34,912.5 |
| IMG_0000 | 15 | 51.00 | 31.01 | 1,581.34 | 1,395.60 | 88.25 | 185.74 | 11.75 | 820 | 287.00 | 272,650.0 |
| Total | — | — | — | 3,141.99 | 2,564.00 | 81.60 | 578.00 | 18.40 | 1,505 | 526.75 | 500,412.5 |

## 15. Worked Example: IMG_2000

### Step 1: Floor count

Detected floors = 4

### Step 2: Facade height

Floor height prior = 3.3 m

Height = 4 × 3.3
       = 13.2 m

### Step 3: Facade width

Width = 28.16 m

This was obtained from the rectified facade mask width and the pixel-to-metre scale.

### Step 4: Total facade area

Area = 13.2 × 28.16
     = 371.76 m²

### Step 5: Usable area

Usable area = 252.89 m²

This comes from counting the usable pixels in the final BIPV mask and converting them to square metres.

### Step 6: Usable percentage

Usable percentage = 252.89 / 371.76 × 100
                  = 68.03%

### Step 7: Excluded area

Excluded area = 371.76 - 252.89
              = 118.87 m²

### Step 8: Panel count

Panel count = floor(252.89 / 1.7)
            = 148 panels

### Step 9: Installed capacity

Capacity = 148 × 350 / 1000
         = 51.8 kWp

### Step 10: Annual energy

Annual energy = 51.8 × 950
              = 49,210 kWh/year

## 16. Just some Key  Explanation To Some Questions

If asked how the model knows the real-world size, the answer is:

The model estimates real-world scale from the detected number of floors and an architectural floor-height prior. It then uses the rectified facade mask to calculate facade width and pixel-to-square-metre conversion. If Google Earth dimensions are supplied, the model can use those dimensions directly as the calibration reference.

If asked whether the result is a direct physical measurement, the answer is:

No. In automatic mode, it is an image-based engineering estimate. It becomes metrically stronger when calibrated with Google Earth or measured facade dimensions. The segmentation determines the usable fraction, while the scale estimation converts that fraction into square metres.

If asked why the usable area is lower than the total facade area, the answer is:

Because windows, doors, balconies, roof/opening areas, and unavailable facade regions are excluded from the installable BIPV surface. The usable area represents the remaining facade wall surface where BIPV modules could potentially be installed.

If asked why panel count is lower than usable area divided exactly by panel area, the answer is:

The panel count uses only complete panels:

floor(usable area / panel area)

Partial panels are not counted.

## 17. Limitations to State Clearly

The current results are suitable for preliminary BIPV facade screening. However, the following limitations should be acknowledged:

- Automatic scale estimation depends on the assumed floor height.

- Strong perspective distortion can affect facade width estimation.

- Missed or false window segmentation affects usable area.

- Obstacle reconstruction may not perfectly recover hidden architecture.

- Annual energy is estimated using a fixed specific yield, not full irradiance simulation.

- Full PVsyst validation should use measured facade dimensions, orientation, shading, and local meteorological data.

## 18. Summary Statement

The pipeline converts a street-level building image into structured BIPV-ready outputs by detecting the facade, removing obstacles, rectifying the facade, segmenting usable and unusable regions, converting mask pixels into square metres, and estimating PV panel capacity and annual energy yield. The calculations are based on explicit geometric formulas and binary mask pixel counts, making the process explainable and reproducible.